[image: image1.png]ecp





    Safety Data Sheet
Date of Issue: 01.06.2021




        Date of Expiry: 01.06.2026
	1: IDENTIFICATION OF THE MATERIAL AND SUPPLIER


	Company Name 

: ECP Limited

Address


: PO Box 34125, Birkenhead, Auckland 0746
Telephone


: +64 9 480 4386

Facsimile


: +64 9 480 4385

Emergency phone number
: 0800 243 622 (24 hours)



	Product

  Sodium Hydroxide  
Code

 47511
CAS No.

UN No.

DG Class/es

Packing group 

1310-73-2

1823

8

II




Recommended use 

: Laboratory Investigations

	2: Hazard’s identification


2.1 GHS Classification

Skin corrosion (Category A), H314

Aquatic toxicity (Acute or Chronic) (Category D), H402

2.2 GHS Label elements, including precautionary statements

Pictogram
[image: image2.png]



Signal word :
Danger
Hazard statement(s)

H314 
Causes severe skin burns and eye damage.

H402 
Harmful to aquatic life.

Precautionary statement(s)

Prevention

P260 
Do not breathe dusts or mists.

P264 
Wash skin thoroughly after handling.

P273 
Avoid release to the environment.

P280 
Wear protective gloves/ protective clothing/ eye protection/ face protection.

Response

P301 + P330 + P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353 IF ON SKIN (or hair): Remove/ Take off immediately all contaminated 
clothing. Rinse skin with water/ shower.

P304 + P340 IF INHALED: Remove victim to fresh air and keep at rest in a position 
comfortable for breathing.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes.

Remove contact lenses, if present and easy to do. Continue rinsing.
P310 
Immediately call a POISON CENTER or doctor/ physician.

P321 
Specific treatment (see supplemental first aid instructions on this label).

P363 
Wash contaminated clothing before reuse.

Storage

P405 
Store locked up.

Disposal

P501 
Dispose of contents/ container to an approved waste disposal plant.

2.3 Other hazards - none

	3: Composition/information on ingredients


3.1 Substances

Synonyms : 

Caustic soda

Formula : 

NaOH

Molecular weight : 
40.00 g/mol

CAS-No. : 

1310-73-2

EC-No. : 

215-185-5

Index-No. : 

011-002-00-6

Hazardous ingredients

	Component
	Classification
	Concentration

	Sodium Hydroxide

	
	8.2 A; 8.3 A; 9.1 D; H314,

H318, H402

Concentration limits:

>= 0.4 %: Met. Corr. 1,

H290; >= 5 %: Skin Corr.

1A, H314; 2 - < 5 %: Skin

Corr. 1B, H314; 0.5 - < 2 %: Skin Irrit. 2, H315; 0.5 - < 2 %: Eye Irrit. 2, H319;
	<= 100 %


	4: First aid measures


4.1 Description of first-aid measures

General advice

First aider needs to protect himself. Show this material safety data sheet to the doctor in

attendance.

If inhaled

After inhalation: fresh air. Call in physician.

In case of skin contact

In case of skin contact: Take off immediately all contaminated clothing. Rinse skin with

water/ shower. Call a physician immediately.

In case of eye contact

After eye contact: rinse out with plenty of water. Immediately call-in ophthalmologist.

Remove contact lenses.

If swallowed

After swallowing: make victim drink water (two glasses at most), avoid vomiting (risk of

perforation). Call a physician immediately. Do not attempt to neutralise.

